### Advantages of Static Relay

The following are the benefits of static relays.

- The static relay consumes very less power because of which the burden on the measuring instruments decreases and their accuracy increases.

- The static relay gives the quick response, long life, high reliability and accuracy and it is shockproof.

- The reset time of the relay is very less.

- It does not have any thermal storage problems.

- The relay amplifies the input signal which increases their sensitivity.

- The chance of unwanted tripping is less in this relay.

- The static relay can easily operate in earthquake-prone areas because they have high resistance to shock.

### Instantaneous Over Current Relay

Construction and working principle of instantaneous over current relay is quite simple.
Here generally a magnetic core is wound by a current coil. A piece of iron is so fitted by hinge support and restraining spring in the relay, that when there is not sufficient current in the coil, the NO contacts remain open. When the current in the coil crosses a preset value, the attractive force becomes enough to pull the iron piece towards the magnetic core, and consequently, the no contacts get closed.

We refer the pre-set value of current in the relay coil as pickup setting current. This relay is referred as instantaneous over current relay, as ideally, the relay operates as soon as the current in the coil gets higher than pick upsetting current. There is no intentional time delay applied. But there is always an inherent time delay which we cannot avoid practically. In practice, the operating time of an instantaneous relay is of the order of a few milliseconds.